X8 - Short throw point source

X Series

Short throw: 15 m

Single element deployment (horizontal or vertical)

Axisymmetric directivity

# X8

## Short description

2-way passive enclosure with 8 in / 1.5 in speaker producing an axisymmetric point source directivity of 100 degrees with consistent off-axis response.

## Description

2-way passive enclosure with a premium 1.5 in diaphragm compression driver coaxially loaded by a 8 in low-mid frequency transducer mounted in a bass-reflex cabinet. The enclosure cabinet is constructed of premium grade Baltic beech and birch plywood. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric. The enclosure integrates an ergonomic handle and a 35 mm pole mount socket. The enclosure provides an angle setting of 35 degrees for stage monitoring. Dedicated accessories ensures wall-mount, ceiling-mount and flown configurations in vertical and horizontal orientations. The enclosure integrates four M10 threaded inserts for external rigging elements and a DIN580-compatible M8 threaded insert for safety. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage (-6 dB):

- Nominal directivity: 100° axisymmetric

- Monitoring angle: 35°

Transducers:

- LF transducer: 8" cone driver

- HF transducer: 1.5" diaphragm compression driver

Acoustics:

- Nominal impedance: 8 Ω

- Usable bandwidth: 60 Hz - 20 kHz (-10 dB), 65 Hz - 20 kHz (-6 dB), 70 Hz - 20 kHz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth): 211 W

- Maximum SPL: 129 dB (Peak level measured at 1 m under free field conditions using pink noise with crest factor 4)

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic beech and birch plywood

- Dimensions (W, H, D): 250 mm, 424 mm, 264 mm / 9.8 in, 16.7 in, 10.4 in

- Weight (net): 12 kg / 26.5 lb

- Connectors: 2 × 4-point speakON

- IP: IP43

- Finish:

  - dark grey brown Pantone 426 C

  - pure white RAL 9010

  - custom RAL code on special order

Product name: X8

Manufacturer: L-Acoustics