X12 - Short throw point source

X Series

Short throw: 15 m

Single element deployment (horizontal or vertical)

Ellipsoid directivity

# X12

## Short description

2-way passive enclosure with 12 in / 3 in speaker producing an ellipsoid directivity of 90 degrees by 60 degrees with consistent off-axis response.

## Description

2-way passive enclosure with a premium 3 in diaphragm compression driver coaxially loaded by a 12 in low-mid frequency transducer mounted in a bass-reflex cabinet. The enclosure cabinet is constructed of premium grade Baltic beech and birch plywood. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric. The enclosure integrates two ergonomic handles and two 35 mm pole mount sockets. The enclosure provides two angle setting of 35 and 55 degrees for stage monitoring. Dedicated accessories ensures wall-mount, ceiling-mount and flown configurations in vertical and horizontal orientations. The enclosure integrates four M10 threaded inserts for external rigging elements and a DIN580-compatible M8 threaded insert for safety. Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage (-6 dB):

- Vertical directivity : 90° symmetric

- Horizontal directivity : 60° symmetric

- Monitoring angles: 35° / 55°

Transducers:

- LF transducer: 12" cone driver

- HF transducer: 3" diaphragm compression driver

Acoustics (preset [X12]):

- Nominal impedance: 8 Ω

- Usable bandwidth: 59 Hz - 20 kHz (-10 dB), 63 Hz - 20 kHz (-6 dB), 70 Hz - 20 kHz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth): 331 W

- Maximum SPL: 136 dB (Peak level measured at 1 m under free field conditions using pink noise with crest factor 4)

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic beech and birch plywood

- Dimensions (W, H, D): 430 mm, 496 mm, 375 mm / 16.9 in, 19.5 in, 14.8 in

- Weight (net): 20 kg / 44.1 lb

- Connectors: 2 × 4-point speakON

- IP: IP43

- Finish:

  - dark grey brown Pantone 426 C

  - pure white RAL 9010

  - custom RAL code on special order

Product name: X12

Manufacturer: L-Acoustics