LA2Xi - AMPLIFIED CONTROLLER FOR INSTALL

System control

# LA2Xi

## Short description

Amplifier with integrated DSP controller – 4 x 360 W at 8 ohms / 4 x 640 W at 4 ohms.

## Description

System amplifier (class D) for installations, with an output power of 4 x 360 W RMS at 8 ohms, A/D converter and integrated DSP processor. Flexible 4 x 4 matrix with 4 inputs and 4 outputs. Outputs can be bridged in pairs with an output power of 2 x 1260 W at 8 ohms. All four output channels can be bridged into one high power output channel of 2550 W. Power supply via SMPS (Switched Mode Power Supply) with power factor correction (PFC). Phoenix terminals for input and output connections. Heat dissipation by temperature-controlled fans. Ethernet connections to pass AVB audio and for remote control and monitoring. Analog audio inputs and digital (AES/EBU) audio inputs with active links. The DSPs process IIR and FIR filters, enable real-time adjustments, contour filters and filters to compensate for air absorption. Protection against thermal overload and over-excursion. Built-in library with factory presets for all manufacturer's loudspeakers and 10 user memories.

Ethernet based control network with up to 1 Gbit/s. Support of digital audio signal transmission via the Milan AVB protocol with seamless redundancy. "Automatic fallback" options are also possible. Design of control networks with up to 253 controller power amplifiers in star, daisy chain or hybrid arrangement. The controller power amplifiers and computers in the network are connected via CAT5e U/FTP or higher, with standard RJ45 connectors. The network management software can be run on both Windows and Mac OS X and is designed to include component load, signal levels, limiter activity, overload, and fault messages. It is also used to manage presets, as well as to adjust input gain, mute, delay correction and filtering. Possible integration into higher-level media control systems by means of software modules.

## Technical requirements

Amplification:

- Output power (no limiter 200 ms sine bursts,1 kHz, < 1% THD, all channels):

  - 4 x 190 W at 16 ohms

  - 4 x 360 W at 8 ohms

  - 4 x 640 W at 4 ohms

- Amplification class: High-efficiency Class D

- Architecture: 4 x 4 Single-Ended Mode or Bridge Mode for: 4 x 3, 4 x 2 and 4 x 1

- Frequency range:  20 Hz - 20 kHz (+/- 0,25 dB, 8 ohms, 60 W output power)

- Distortion THD+N: < 0.1% (20 Hz - 10 kHz, 8 ohms, 60 W output power)

- Output dynamic range: > 113 dB (20 Hz - 20 kHz, 8 ohms, A-weighted, digital input)

- Noise level < -78 dBV (20 Hz - 20 kHz, 8 ohms, A-weighted, digital input)

- Amplification gain: 32 dB

- Channel separation: > 80 dB (1 kHz, 8 ohms, 60 W)

- Output delay: adjustable from 0 to 1000 ms

DSP:

- Gen. 4 Dual SHARC 32-bit, floating point, 96 kHz Sampling frequency

- Routing inputs/outputs: flexible 4 x 4 routing and summation matrix

- Latency for analog and digital inputs:

  - In standard operating mode: 3.84 ms

  - In low latency operating mode: 0.76 ms

Analog input:

- 4 channels

- Input impedance: 22 kOhm (balanced)

- Maximum input level: 22 dBu (balanced, THD 1%)

- 32-bit A/D converter (121 dB dynamic range, 20 Hz - 20 kHz, A-weighted)

Digital input:

- 4 channels with active link and failsafe relay

- Standards: AES/EBU (AES3) or S/PDIF

- Supported Sampling frequency: 44.1 kHz to 192 kHz

- Word length: 16 to 24 bits

Sample Rate Converter (SRC)

- Sampling frequency: 96 kHz

- Word length: 24 bits

- Dynamic range: 140 dB

- Distortion THD+N: < -120 dBFS

Input gain:

- Range: -12 dB to + 12 dB

- Steps: 0.1 dB

AVB input

- MILAN™-certified

- 4 channels 48 kHz / 96 kHz from 1 stream of up to 8 channels

- Standards: Ethernet AVB IEEE 802.1BA-2011, Number: 1, Class: A

- Latency: 2 ms max

- Formats: AAF PCM32 or IEC 61883-6 AM824 at 48 kHz or 96 kHz

- Clock: synchronized on clock of the connected AVB input stream

Connectors:

- Inputs: 4 male 3-point terminal blocks

  - 2 for analog

  - 2 for AES/EBU or analog

- Input links: 2 male 3-point terminal block for AES/EBU or analog

- dual-port Ethernet Gigabit interface with 2 x RJ45/etherCON 1 Gb/s

- General Purpose I/O (GPIO): 4 GPIO, isolated optocoupler inputs, isolated relay contacts

- Mains rating: 100 V AC - 240 V AC +/- 10%, 50 Hz - 60 Hz

- Nominal current requirements: 10 A for 200 V AC - 240 V AC

- Backup power circuit for the DSP: 24 V DC 2-point terminal block

- Loudspeaker outputs: 2 x 4-point terminal blocks

Display:

- Information LED: power, network, status

- Output channels LED: signal, limit, clip

Third-party management solutions:

- QSC Q-SYS™

- Crestron®

- Extron®

- SNMP (Simple Network Management Protocol)

Operating conditions:

- Operating temperature: between -5 °C / 23 °F and 50 °C / 122 °F

- Cooling system: 2 integrated, temperature-controled fans

Physical data:

- Dimensions (W, H, D): 483 mm, 44.45 mm, 265 mm / 19 in, 1.75 in, 10.43 in

- Rack mounting: 1U

- Weight (net): 4.4 kg / 9.7 lb

- IP: IP2x

- Finish: Black

Product name: LA2Xi

Manufacturer: L-Acoustics