# LA7.16 - AMPLIFIED CONTROLLER

System control

# LA7.16

## Short description

Amplifier with integrated DSP controller – 16 x 1300 at 8 ohms / 16 x 1100 W at 4 ohms.

## Description

System amplifier (class D) with an output power of 16 x 1300 at 8 ohms, A/D converter, and integrated DSP processor. Flexible 16 x 16 matrix with 16 inputs and 16 outputs and SC32 speaker connector. Power supply via SMPS (Switched Mode Power Supply) with power factor correction (PFC) and 24V DC backup supply input. Advanced DSP algorithms optimize real-time performance from available SMPS and amplifier resources. Heat dissipation by DSP-controlled fans. Analog audio input, digital (AES/EBU) audio input with active links, and GPIO connections using Phoenix terminal block. Ethernet connections to pass Milan-AVB audio and for remote control and monitoring. The DSPs process IIR and linear phase FIR filters enable real-time adjustments, contour filters, and filters to compensate for air absorption. Protection against thermal overload and over-excursion. Built-in library with factory presets for all manufacturers' loudspeakers and 10 user memories.

Ethernet-based control network with up to 1 Gbit/s. Support of digital audio signal transmission via the Milan-AVB protocol with seamless redundancy. "Automatic fallback" options are also possible. Design of control networks with up to 253 controller power amplifiers in star, daisy chain, or hybrid arrangement. The controller power amplifiers and computers in the network are connected via CAT5e U/FTP or higher, with standard RJ45 connectors. The network management software can be run on both Windows and Mac OS X and is designed to include component load, signal levels, limiter activity, overload, and fault messages. It is also used to manage presets, as well as to adjust input gain, mute, delay correction, and filtering. Integration into higher-level media control systems is possible by means of software modules.

## Technical requirements

Amplification:

- Output power (12 dB Crest Factor 2 ms sine bursts, 1 kHz, < 1% THD, all channels):

  - 16 x 700 W at 16 ohms

  - 16 x 1300 W at 8 ohms

  - 16 x 1100 W at 4 ohms

- Amplification class: High-efficiency Class D

- Architecture: 16 x 16

- Frequency range:  20 Hz - 20 kHz (+/- 0.05 dB)

- Distortion THD+N: < 0.05% (20 Hz - 20 kHz, 8 ohms, 60 W)

- Output dynamic range: > 119 dB (20 Hz - 20 kHz, 8 ohms, A-weighted)

- Noise level < -78 dBV (20 Hz - 20 kHz, 8 ohms, A-weighted)

- Amplification gain: 32 dB

- Channel separation: > 65 dB (1 kHz, 8 ohms, 60 W)

- Output delay: adjustable from 0 to 1000 ms

DSP:

- Gen. 5 Dual SHARC 32-bit, floating-point, 96 kHz Sampling frequency

- Routing inputs/outputs: flexible 16 x 16 routing and summation matrix

- Latency for analog and digital inputs:

  - In standard operating mode: 3.84 ms

  - In low latency operating mode: 1.18 ms

AVB input

- 128 channels 48 kHz / 96 kHz from 16 streams of up to 8 channels

- Standards: Ethernet AVB IEEE 802.1BA-2011, Number: 1, Class: A

- Latency: 2 ms max

- Formats: AAF PCM32 or IEC 61883-6 AM824 at 48 kHz or 96 kHz

- Clock: synchronized on clock of the connected AVB input stream

- MILAN™-certified

Analog input:

- 1 channel with active link and failsafe relay

- Input impedance: 22 kOhm (balanced)

- Maximum input level: 22 dBu (balanced, THD 1%)

- 32-bit A/D converter (121 dB dynamic range, 20 Hz - 20 kHz, A-weighted)

Digital input:

- 2 channels with active link and failsafe relay

- Standards: AES/EBU (AES3)

- Supported Sampling frequency: 44.1 kHz to 192 kHz

- Word length: 16 to 24 bits

Sample Rate Converter (SRC)

- Sampling frequency: 96 kHz

- Word length: 24 bits

- Dynamic range: 140 dB

- Distortion THD+N: < -120 dBFS

Input gain:

- Range: -12 dB to + 12 dB

- Steps: 0.1 dB

Connectors:

- Dual-port Ethernet Gigabit interface with 2 x RJ45/etherCONTM 1 Gb/s

- 12-point terminal block utilizing:

  - Inputs: 3-points for AES/EBU or analog

  - Input links: 3-points for AES/EBU or analog

  - General Purpose I/O (GPIO): 3 GPIO, isolated optocoupler inputs, isolated relay contacts 4-points

  - Backup power circuit for the DSP: 24 V DC 2-point terminal block

- Mains rating: 100 V AC - 240 V AC +/- 10%, 50 Hz - 60 Hz

- Nominal current requirements: 16 A for 200 V AC - 240 V AC

- Loudspeaker outputs:

  - 1 x SC32 female connector for Out 1-16

Display:

- Information LED: power, network, status

- Input channels LED: signal, limit, clip

- Output channels LED: signal, limit, clip

Third-party management solutions:

- Q-SYS™

- Crestron®

Operating conditions:

- Operating temperature: between -5 °C / 23 °F and 50 °C / 122 °F

- Cooling system: 5 integrated, DSP-controlled fans

Physical data:

- Dimensions (W, H, D): 483 mm, 88 mm, 510 mm / 19 in, 3.5 in, 20.1 in

- Rack mounting: 2U

- Weight (net): 15.8 kg / 34.8 lb

- IP: IP2x

- Finish: Black

Product name: LA7.16

Manufacturer: L-Acoustics