A15 - Medium throw line source

A Series

Medium throw: 45 m

Deployment in horizontal / vertical lines

Incremental coverage with fixed inter-element angles

# A15 Focus

## Short description

2-way passive enclosure with 15 in / 3 in speaker – 10-degree enclosure angle and adjustable waveguide directivity for assembling enclosures to achieve a coherent wavefront line source.

## Description

2-way passive enclosure with a weather resistant premium 15 in transducer and high output 3 in diaphragm compression driver coupled to a cylindrical waveguide with user-adjustable directivity. The enclosure cabinet is constructed of premium grade Baltic beech and birch plywood. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric. The enclosure integrates two ergonomic handles, and panel corner stops for enclosure protection. The Flush-fitting hardware allows for rigging and combining with other loudspeakers of the same family to achieve a coherent wavefront line source. The line source directivity is equal to N x 10 degree (where N is the number of enclosures in the array). Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage (-6 dB):

- Enclosure directivity: 10°

- Waveguide directivity: 70° / 110° symmetric or 90° asymmetric

Transducers:

- LF transducer: 15" cone driver

- HF transducer: 3" diaphragm compression driver

Acoustics:

- Nominal impedance: 8 Ω

- Usable bandwidth: 41 Hz - 20 kHz (-10 dB), 44 Hz - 20 kHz (-6 dB), 47 Hz - 20 kHz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth): 446 W

- Maximum SPL: 144 dB (Peak level measured at 1 m under free field conditions using pink noise with crest factor 4)

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic beech and birch plywood

- Dimensions (W, H, D): 762 mm, 427 mm, 490 mm / 30 in, 16.8 in, 19.3 in

- Weight (net): 35 kg / 77 lb

- Connectors: 2 × 4-point speakON

- IP: IP55

- Finish: dark grey brown Pantone 426 C

Product name: A15 Focus

Manufacturer: L-Acoustics

# A15 Wide

## Short description

2-way passive enclosure with 15 in / 3 in speaker – 30-degree enclosure angle and adjustable waveguide directivity for assembling enclosures to achieve a coherent wavefront line source.

## Description

2-way passive enclosure with a weather resistant premium 15 in transducer and high output 3 in diaphragm compression driver coupled to a cylindrical waveguide with user-adjustable directivity. The enclosure cabinet is constructed of premium grade Baltic beech and birch plywood. The enclosure front is protected by a coated steel grill and an acoustically transparent fabric. The enclosure integrates two ergonomic handles, and panel corner stops for enclosure protection. The Flush-fitting hardware allows for rigging and combining with other loudspeakers of the same family to achieve a coherent wavefront horizontal or vertical line source. The line source directivity is equal to N x 30 degree (where N is the number of enclosures in the array). Exclusively driven and protected by a proprietary amplified controller through a dedicated preset.

## Technical requirements

Coverage (-6 dB):

- Enclosure directivity: 30°

- Waveguide directivity: 70° / 110° symmetric or 90° asymmetric

Transducers:

- LF transducer: 15" cone driver

- HF transducer: 3" diaphragm compression driver

Acoustics:

- Nominal impedance: 8 Ω

- Usable bandwidth: 42 Hz - 20 kHz (-10 dB), 47 Hz - 20 kHz (-6 dB), 52 Hz - 19 kHz (-3 dB)

- RMS power handling (Calculated using the mean impedance measured on the usable bandwidth): 446 W

- Maximum SPL: 141 dB (Peak level measured at 1 m under free field conditions using pink noise with crest factor 4)

Physical data:

- Acoustical load: bass-reflex

- Cabinet: premium grade Baltic beech and birch plywood

- Dimensions (W, H/h, D): 762 mm, 424 mm/182 mm, 494 mm / 30 in, 16.7 in/8 in, 19.4 in

- Weight (net): 33 kg / 73 lb

- Connectors: 2 × 4-point speakON

- IP: IP55

- Finish: dark grey brown Pantone 426 C

Product name: A15 Wide

Manufacturer: L-Acoustics